# Scag Tiger Cat 2: Comprehensive Spindle, Pulley, and Maintenance Guide

This document provides a detailed technical roadmap for replacing all three spindle assemblies and pulleys on a Scag Tiger Cat 2, along with a total machine maintenance schedule based on official Scag protocols.

## 1. Technical Specifications & Lubrication

Proper lubrication is the primary factor in extending the life of new spindles. According to Scag maintenance standards, the following intervals must be observed to maintain peak performance:

| Component | Interval | Lubricant Type |
| --- | --- | --- |
| Cutter Deck Spindles | 40 Hours / Weekly | Spindle Grease |
| Caster Wheel Pivot | 100 Hours / Bi-Weekly | Chassis Grease |
| Brake Actuator Levers | 100 Hours / Bi-Weekly | Chassis Grease |

## 2. The Spindle & Pulley Replacement Workflow

### Pre-Installation Checklist

- Safety: Disconnect spark plug wires to prevent accidental startup.

- Deck Prep: Scrape the underside of the Velocity Plus deck to bare metal. This ensures maximum airflow and prevents the new spindles from seating on debris.

- Inspection: Check the deck reinforcement plate for "mushrooming" or cracks before mounting new hardware.

### Installation Tips & Tricks

- The "Anti-Seize" Rule: Apply a thin layer of anti-seize to the spindle shaft before sliding the pulley on. This prevents future seizing.

- Woodruff Key Alignment: Use a dab of grease to hold the woodruff key in place during pulley installation to prevent it from tipping.

- Torque Pattern: Tighten spindle-to-deck bolts in a star pattern to 30-35 ft-lbs to ensure a perfectly level mounting.

- Pulley Torque: Torque the top pulley nut to 55-65 ft-lbs.

### Critical Measurements

- Pulley Height: Measure from the deck shell to the pulley rim to ensure the belt remains co-planar across all three spindles.

- Blade Tracking: After installation, rotate blades so tips meet; they should be within 1/8" of each other in height.

- Deck Pitch: Ensure the front of the deck is 1/8" to 1/4" lower than the rear for optimal discharge.

## 3. General Machine Maintenance Schedule

While the deck is serviced, perform these high-priority engine and drive system tasks:

- Engine Oil & Filter: Change every 100 hours.

- Fuel Filter: Replace every 100 hours.

- Hydraulic System: Replace hydraulic oil and filters every 500 hours.

- Tire Pressure: Maintain 12 PSI (Rear) and 25 PSI (Front) for a level cut.

- Belt Tension: Check drive belts for "checking" or cracks. The belt should deflect 1/2" with 10 lbs of pressure.

Note: Always consult your specific Scag Illustrated Parts Manual for model-year specific part numbers (48", 52", or 61" decks).