# Scag Tiger Cat 2: Complete Spindle, Pulley, and Maintenance Master Guide

Line Spacing: 1.15

## 1. Required Tools & Supplies

To perform this overhaul professionally, ensure you have the following tools on hand:

- Sockets & Wrenches: 1/2", 9/16", 5/8", and 3/4" (Standard and Metric sets recommended).

- Torque Wrench: Essential for Blade Bolts (75-100 ft-lbs) and Pulley Nuts (55-65 ft-lbs).

- Impact Wrench: 1/2" drive (cordless or pneumatic) for removing stubborn deck hardware.

- Pulley Removal Hardware: Two Grade 8 hex head bolts matching the thread of the pulley hub holes (the "Bolt Trick").

- Maintenance Supplies:

  - High-Temp Lithium Grease (Scag Premium Spindle Grease p/n 486258).

  - Anti-Seize Lubricant (for spindle shafts).

  - Wire Brush and Heavy-Duty Putty Knife (for deck cleaning).

## 2. The Spindle & Pulley Replacement Process

### Pre-Removal Measurements

Before removing old units, record these distances to ensure factory-spec alignment:

- Pulley Deck Height: Measure from the top of the deck shell to the top of the pulley rim.

- Blade-to-Deck Gap: Measure from the blade tip to the bottom edge of the deck rim.

### Installation Tips & Tricks

| Action | Pro-Tip |
| --- | --- |
| Deck Prep | Scrape the underside to bare metal. Ensure the mounting holes are not "mushroomed" upward. |
| Woodruff Key | Apply a dab of grease to the back of the key to "glue" it into the slot so it doesn't tip during pulley installation. |
| Anti-Seize | Apply to the spindle shaft before seating the pulley to prevent future galvanic corrosion. |
| Star Pattern | Tighten the 4-6 spindle mounting bolts in a star pattern to 30-35 ft-lbs. |

## 3. Comprehensive Maintenance & Lubrication

While the deck is disassembled, address the following service points per the Scag Operator's Manual:

### The "Hidden" Grease Map

- New Spindles: Grease immediately until you feel resistance or see a tiny weep.

- Caster Yokes: Grease the vertical shafts on the front wheels (every 40-100 hours).

- Pump Drive Idler: Located under the seat; ensure the pivot arm moves freely.

- Deck Lift Pivot: Grease the main cross-shaft to keep height adjustments smooth.

### General Machine Health

- Engine Oil: Change every 100 hours (10w-30 or 15w-40).

- Hydro System: Check expansion tanks. If oil is "Chocolate Milk" or "Black," change filters and fluid (typically 500-hour interval).

- Tire Pressure: Set Rears to 12 PSI and Fronts to 25 PSI. This is vital for a level cut.

## 4. Final Validation Checklist

- Blade Tracking: Rotate blades so tips meet. They must be within 1/8" of each other.

- Deck Pitch: Ensure the front of the deck is 1/8" to 1/4" lower than the rear.

- Belt Tension: Verify the belt deflects 1/2" under 10 lbs of pressure.

- Hot Check: After 15 minutes of mowing, verify spindle housings are warm but not excessively hot.